JNC

TCP-485

Port Serial to Ethernet Converter or

Modbus Gateway

[Features]

| - 1 set of RS232/485/422 to Ethernet converter - Modbus TCP to Modbus RTU converter - Use HTTP browser or Utility to set - Built-in password protection function in the setting interface - Isolated power and low power consumption - Low cost and high reliability |  |
| --- | --- |

V    JNC TECHNOLOGY CO., LTD.  TEL:06-3110008  FAX:06-3110522   E-mail: jnc.jnc@msa.hinet.net

Copyright © 2016  JNC TECHNOLOGY CO., LTD.                         JNC reserve the right to change the specifications.

[Specifications]                                     [Dimension] ｍｍ

V    JNC TECHNOLOGY CO., LTD.  TEL:06-3110008  FAX:06-3110522   E-mail: jnc.jnc@msa.hinet.net

Copyright © 2016  JNC TECHNOLOGY CO., LTD.                         JNC reserve the right to change the specifications.

V    JNC TECHNOLOGY CO., LTD.  TEL:06-3110008  FAX:06-3110522   E-mail: jnc.jnc@msa.hinet.net

Copyright © 2016  JNC TECHNOLOGY CO., LTD.                         JNC reserve the right to change the specifications.

V    JNC TECHNOLOGY CO., LTD.  TEL:06-3110008  FAX:06-3110522   E-mail: jnc.jnc@msa.hinet.net

Copyright © 2016  JNC TECHNOLOGY CO., LTD.                         JNC reserve the right to change the specifications.

V1.00    JNC TECHNOLOGY CO., LTD.  TEL:06-3110008  FAX:06-3110522   E-mail: jnc.jnc@msa.hinet.net

Copyright © 2016  JNC TECHNOLOGY CO., LTD.                         JNC reserve the right to change the specifications.

P1

| Power Supply | Input Voltage | 24VDC ◆Power consumption:2W |
| --- | --- | --- |
|  | Power Connector | Terminals |
|  | Isolation | 2KV |
| Environment | Working Temp. | 0~60℃ / 5 ~ 95%RH non-condensing |
|  | Storage Temp. | -40~85℃ |
|  | Height | Up to 2000m (795hPa) peak demand |
| Ethernet Interface | Communication Method | Modbus TCP |
|  | Speed | 10/100Mbps，automatic MDI/MDIX |
|  | Transformer isolation protection | 1500VRMS |
|  | Quantity of mesh holes | 1 |
| Conversion Interface | Communication Method | Modbus RTU/ASCII Slave/Master |
|  | ESD protection | All signals are 15 kV |
|  | RS-485 data transmission | Two-way |
|  | Quantity of conversion holes | 1 |
| Convert Communication Parameters | Bau rate | 300bps~921.6kbps |
| Convert Signal | RS-232 | TxD，RxD，RTS，CTS，DTR，DSR，DCD |
|  | RS-422 | Tx+，Tx-，Rx+，Rx- |
|  | RS-485-2w | Data+， Data - |
| Physical Condition | Product Size (mm) | No hanging ears:25 x 69 x 102 Hanging ears:25 x 94 x 102 (W× H× D) |
|  | Weight | 300g |
| Shell | Material | PC |
| Software | Setting method | HTTP Web Console， Windows Setup Utility |
|  | Utilities | Setup Utility for Windows |